LAND CORNER RECORDATION CERTIFICATE

Filing Requirement of Act 74, Mich. PA 1970

| For Corners In: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Alpena |  |  |  |  |  |  |  |  |  |  |  | Located In |  |  |  |  |  |  | Corner Code # |  |  |  |
| (County) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. Public Land Survey |  |  |  |  |  |  |  |  |  |  |  |  | T | 30N |  |  | R | 8E |  |  | B-9 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | T | 30N |  |  | R | 8E |  |  | C-9 |  |
| 2. Property Controlling in Section |  |  |  |  |  |  | S |  |  |  |  |  | T |  |  |  | R |  |  |  |  |  |
|  |  |  |  |  |  |  | S |  |  |  |  |  | T |  |  |  | R |  |  |  |  |  |
| 3. Miscellaneous Property in Section |  |  |  |  |  |  | S |  |  |  |  |  | T |  |  |  | R |  |  |  |  |  |
|  |  |  |  |  |  |  | S |  |  |  |  |  | T |  |  |  | R |  |  |  |  |  |
| 4. Lot No: |  |  |  |  |  |  | Recorded Plat: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5. Private Claims: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| I, , in a field survey on September 16, 2015 do hereby state that under requirements of PA 74, Michigan PA of 1970, the corner points mentioned in lines 1 and 2 above were in conformance with regulations and rules therefore as required in the current manual of survey instructions of the United States Department of the Interior, Bureau of Land Management or by a decree of a Court of Law and/or that the corner points mentioned in lines 3, 4 and 5 above were in conformance with the rules of the Michigan Board of Land Surveyors or by a Decree of a Court of Law; established, re-established, monumented, recovered, found as expressed below: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE: Not more than 2 corners, all in the same town and range, may be recorded on this certificate. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| A B C D E F G H I J K L M M 1 2 3 4 5 6 7 8 9 10 11 12 13 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 1 3 6 5 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 9 8 7 11 12 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 14 17 15 18 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 22 21 20 23 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 25 26 29 27 30 28 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 36 35 32 34 31 33 |  |  |  |  |  |  |  |  |  |  |  |  |  |

| A. Description of original monument and accessories and/or subsequent restoration: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B-9 – ¼ Corner common to sections 19 & 30, T30N,R8E 1839 – GLO – Lewis Claison set post cor. Secs. 19 & 30 Witness trees inaccessible. Sept. 1918 - Department of Conservation Land Surveyor Marcus Schaaf RLS# 3104 set a monument by single proportionate measurement. |  |  |  | C-9 – Section Corner common to sections 19,20,29 & 30,T30N,R8E 1839 – GLO – Lewis Claison set post cor. Secs. 19,20,29 & 30,T30N,R8E Wits. Cedar 11” S74°W 23 links Cedar 11” N28°E 20 links Sept. 1918 - Department of Conservation Marcus Schaaf RLS# 3104 set a monument by proportionate measurement. No witnesses given. |  |  |  |
| B. Description of corner evidence found and/or method applies in restoring or re-establishing corner: |  |  |  |  |  |  |  |
| B-9 – ¼ Corner common to sections 19 & 30, T30N,R8E Found parts of M. Schaaf concrete monument hit by equipment the pipe was gone. Set a 1” x 36” pipe with Standard DNR Cap by single proportionate measurement. Monument fits well with Marcus Schaaf’s measurements East and West to found corners. Accepted monument as the best available evidence of the original Government corner. |  |  |  | C-9 – Section Corner common to sections 19,20,29 & 30,T30N,R8E Found M. Schaaf 2” pipe in concrete in the center of an old road bed intersection. The distance West fits well with M. Schaaf measurement West. |  |  |  |
| C. Description of monument for corner and accessories established to perpetuate locating the position of the corner: |  |  |  |  |  |  |  |
| B-9 – ¼ Corner common to sections 19 & 30, T30N,R8E 1”x36” galvanized pipe with Standard DNR Cap Witnesses: N54°E 37.43’ to set N&T on NW side 8” Cedar S04°W 11.71’ to set N&T on E side 11” Cedar S70°W 25.04’ to set N&T on S side 12” Cedar N45°W 17.33’ to set N&T on NE side 6” Cedar Center East – West old road bed North 2’ Set Blue Post 1’ North |  |  |  | C-9 – Section Corner common to sections 19,20,29 & 30,T30N,R8E Added P.S. Cap #33981 to found 2” pipe in concrete Witnesses: N43°E 39.02’ to set N&T on E side 12” Balsam S52°E 29.60’ to set N&T on NE side 6” Balsam S55°W 16.98’ to set N&T on NW side Balsam S79°W 30.42’ to set N&T on N side 6” Balsam N34°W 38.38’ to set N&T on SW side 11” Balsam Set Blue Post w/loc. tag 1’ North |  |  |  |
|  | Signed by: |  |  |  | Date: |  |  |
|  | Surveyor's Michigan License No.: |  |  |  |  |  |  |