|  | Michigan Department of Natural Resources, Forest, Mineral and Fire Management Timber Sale Prescription |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GENERAL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date 8/1/2006 |  |  |  |  |  |  |  |  |  | Forest, Mineral and Fire Management Unit Baraga |  |  |  |  |  |  |  |
| Timber Sale Number (if applicable) 11-019-07-01 |  |  |  |  |  |  |  |  |  | Sale Name (or prescription name) Kidney Hdwd |  |  |  |  |  |  |  |
| LOCAL CONTACT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name Jason Mittlestat |  |  |  |  |  |  |  |  |  | Telephone (906) 353 - 6651 |  |  |  |  |  |  |  |
| Email Address mittlesj@michigan.gov |  |  |  |  |  |  |  |  |  | FAX Number (906) 353 - 7464 |  |  |  |  |  |  |  |
| Map of Project Area Attached |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LEGAL DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T51N R 39W, 40W Section(s) 39W: Sec 30, 40W: Sec 24, 25 Description See Map |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Year of Entry: 2007 Compartment(s): 70 Stand Number(s): 10, 12, 13, 17, 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| THIS TIMBER SALE CONTRACT IS BASED ON THE FOLLOWING ACREAGE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Estimated Acres: 170 Source: X OI GPS Other Payment will be made on the basis of these estimated acres. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TREATMENT & OBJECTIVE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STAND # |  | COVER TYPE |  | ACRES |  | BA |  |  | TREATMENT |  |  |  | MANAGEMENT OBJECTIVE |  |  |  |  |
| 10 |  | M6 |  | 59 |  | 120 |  |  | Selection |  |  |  | Northern Hdwd |  |  |  |  |
| 12 |  | M6 |  | 15 |  | 120 |  |  | Selection |  |  |  | Northern Hdwd |  |  |  |  |
| 13 |  | M6 |  | 12 |  | 120 |  |  | Selection |  |  |  | Northern Hdwd |  |  |  |  |
| 17 |  | M9 |  | 9 |  | 150 |  |  | Selection |  |  |  | Northern Hdwd |  |  |  |  |
| 18 |  | M6 |  | 75 |  | 120 |  |  | Selection |  |  |  | Northern Hdwd |  |  |  |  |
| PRESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Mark trees to be cut. Favor the best future crop tree in place, regardless of species. - Mark payment unit lines in yellow. Stand #18 will be 2 payment units,stand 10 will be 2 units. - Leave active den and nest trees. Favor oak, hemlock, and pine. - The objective is to grow N. Hdwd to a 22" DBH. - Retain all hemlock and pine unless removal is needed for operability. - Release oak on three sides to a residual BA of 60 sq ft. - Where there is more than 30 sq ft of hemlock mark down to 100 sq ft. - GPS all roads within the sale area. GPS corners. - Label sale name and unit numbers at all unit & road intersections - Label units when unit line is adjacent to roads. Labeling is to be done in red aerisol paint. Access Successful bidder is responsible for obtaining permission to cross any private lands. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DNR PREPARATION WORK TO BE DONE PRIOR TO CONTRACT WORK |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ESTIMATED DATE |  |  |
| Survey |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 11-01-06 |  |  |
| CONTRACT WORK CAN BEGIN X Immediately Date: CONTRACT WORK MUST BE COMPLETED BY Date Lines by: 6-01-07 Marking by 8-31-07 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PAINT LINE WORK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This is included in the bid This is not included in the bid Paint line work to be performed: (See attached map for locations) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TYPE OF LINE |  |  |  |  | WORK TO BE DONE |  |  | NOT APPLICABLE |  |  |  | PAINT COLOR |  |  |  |  |  |
| Private boundary |  |  |  |  |  |  |  |  |  |  |  | Blue |  |  |  | Other: See ** |  |
| Sale boundary |  |  |  |  |  |  |  |  |  |  |  | Red |  |  |  | Other: See ** |  |
| Sale cutting unit |  |  |  |  |  |  |  |  |  |  |  | Yellow |  |  |  | Other: See ** |  |
| Stand type line |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Other: |  |
| Exclusions to mark and why |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Standards for marking lines against private land All lines must be flagged w/ pink ribbon, and painted so as to be visible from all vantage points along the line. GPS can be used to approximate line placement, but the final line must essentially be straight, and extend corner to corner. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| AREA CALCULATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This is included in the bid This is not included in the bid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UNIT |  |  | METHOD |  |  |  |  |  |  |  |  |  |  |  | STANDARD |  |  |
| Sale |  |  | GPS String Chain Other |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Payment Unit |  |  | GPS String Chain Other |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Stand |  |  | GPS String Chain Other |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Special Instructions: Provide corrected GPS files compatable with Pathfinder software. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TIMBER CRUISING SPECIFICATIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This is included in the bid X This is not included in the bid Required Basal Area Factor: 10 20 Other: Cruise Line Directions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRUISING UNITS |  |  |  |  |  |  | NUMBER OF PLOTS PER ACRE |  |  |  |  |  |  | SPACING (CHAINS) |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |
| TOTAL NUMBER OF CRUISE POINTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cruise Special Instructions: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Unless otherwise specified, the tally sheets used must be those supplied by the Forest, Mineral and Fire Management Unit. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| TIMBER MARKING SPECIFICATIONS |  |  |  |
| --- | --- | --- | --- |
| This is included in the bid This is not included in the bid |  |  |  |
| Precise specifications to be delineated at pre-work meeting with DNR staff |  |  |  |
| Total Residual BA (Basal Area): 70 (Minimum 60 to Maximum 80) |  |  |  |
| TREATMENT |  | NUMBER | SIZE |
| Regeneration Gaps Per Acre |  | n/a |  |
| Girdled Trees Per Acre |  | n/a |  |
| PRODUCT | MARKING SYMBOL | MINIMUM DBH | TOP DIB |
| Sawlogs | slash on 2 sides w/dots | 10" | 8" |
| Sawbolts |  |  |  |
| Pulpwood | slash on 2 sides. see * | 6" | 4" |
| TYPE OF WORK | WORK TO BE DONE | NOT APPLICABLE | PAINT COLOR |
| Trees marked to leave |  |  | Green Other |
| Trees marked to cut |  |  | Orange Other |
| TALLY INTENSITIES |  |  |  |
| SPECIES / PRODUCT |  | RATIO |  |
| Hardwood pulp |  | 1 : 20 |  |
| Softwood pulp |  | 1 : 10 |  |
| Aspen (all as pulp) |  | 1 : 10 |  |
| Sawlogs |  | 1 : 1 |  |
| SPECIES TO FAVOR Oak, Hemlock, White Pine, Cedar |  |  |  |
| SPECIES TO FAVOR |  |  |  |
| SPECIAL MARKING INSTRUCTIONS Put upper marks on two opposite sides of trees 6' up. The contractor must furnish the DNR with plot data showing residual BA/Ac for payment units to be checked (Self Admin). This is due before The DNR checks the marking. The contractor is responsible for checking all workers on the sale. Final marking details are to be discussed at a prework conference. * Every tally pulpwood tree should have a slash for every tallied 8' stick. ** Line marks must be atleast 6' high, be flagged w/pink ribbon, and painted so as to be visible from all vantage points along the line. |  |  |  |
| Unless otherwise specified, the tally sheets used must be those provided by the Forest, Mineral and Fire Management Unit. |  |  |  |
| RESTRICTIONS - Forest, Mineral and Fire Management Division must to be able to identify who did what on the timber sale. - The contractor may not have more than two markers in one payment unit without prior approval. - Each marker should have an assigned area to mark (not strips). |  |  |  |
| PAINT - Call Jason Mittlestat at (906) 353 - 6651, to make arrangements for paint. - Paint will be supplied from Baraga OSC. - Contractor’s work schedule must coincide with the DNR work schedule for paint supply. - The DNR will not be available on weekends or evenings. |  |  |  |
| DELIVERABLES - A list of names that marked or cruised each payment unit. - A summary of paint used (in quarts). - Tally summaries for each payment unit (one summary, regardless of the number of markers) and all of the original tally sheets. - GPS files for any GPS work. - 30% of bid amount will be paid upon line work completion and approval. |  |  |  |