|  | MICHIGAN DEPARTMENT OF NATURAL RESOURCES FOREST, MINERAL AND FIRE MANAGEMENT |
| --- | --- |

TIMBER SALE PRESCRIPTION

| GENERAL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date 9/8/06 |  |  |  |  |  |  |  |  |  | Forest, Mineral and Fire Management Unit GAYLORD |  |  |  |  |  |  |  |  |
| Timber Sale Number (if applicable) |  |  |  |  |  |  |  |  |  | Sale Name (or prescription name) Compartment 41 Stands 32,33,34,37 & 40 |  |  |  |  |  |  |  |  |
| LOCAL CONTACT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Telephone Number (989) 732 - 3541 |  |  |  |  |  |  |  |  |  | FAX Number (989) 731 - 6214 |  |  |  |  |  |  |  |  |
| Name GREGORY GATESY |  |  |  |  |  |  |  |  |  | Email Address gatesyg@michigan.gov |  |  |  |  |  |  |  |  |
| Map of Project Area Attached |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LEGAL DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T33N R4W Section 31: S1/2 AND S1/2N1/2, Section 32: SW1/4SW1/4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T33N R4W Section 33: N1/2SW1/4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T32N R4W Section 6: NW1/4NE1/4 & NE1/4NW1/4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Year of Entry: 2007 Compartment(s):41 Stand Number(s): 32, 33, 34, 37 and 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| THIS TIMBER SALE CONTRACT IS BASED ON THE FOLLOWING ACREAGE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Estimated Acres: 256 Source: OI GPS X Other ArcView digitized map Payment will be made on the basis of these estimated acres. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TREATMENT & OBJECTIVE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STAND # | COVER TYPE |  | ACRES |  |  | BA |  |  | TREATMENT |  |  |  |  | MANAGEMENT OBJECTIVE |  |  |  |  |
| 7 compt 43 | M9 |  | 16 |  |  | 130 |  |  | SELECTION |  |  |  |  | NORTHERN HARDWOODS |  |  |  |  |
| 8 compt 43 | M6 |  | 17 |  |  | 100 |  |  | SELECTION |  |  |  |  | NORTHERN HARDWOODS |  |  |  |  |
| 32 compt 41 | M6 |  | 68 |  |  | 90 |  |  | SELECTION |  |  |  |  | NORTHERN HARDWOODS |  |  |  |  |
| 33 compt 41 | M6 |  | 27 |  |  | 120 |  |  | SELECTION |  |  |  |  | NORTHERN HARDWOODS |  |  |  |  |
| 34 compt 41 | M6 |  | 82 |  |  | 100 |  |  | SELECTION |  |  |  |  | NORTHERN HARDWOODS |  |  |  |  |
| 37 compt 41 | M6 |  | 16 |  |  | 130 |  |  | SELECTION |  |  |  |  | NORTHERN HARDWOODS |  |  |  |  |
| 40 compt 41 | M6 |  | 30 |  |  | 100 |  |  | SELECTION |  |  |  |  | NORTHERN HARDWOODS |  |  |  |  |
| PRESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. Reduce basal area to 80 sq ft. Access: Chandler Road |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DNR PREPARATION WORK TO BE DONE PRIOR TO CONTRACT WORK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ESTIMATED DATE |  |  |
| CONTRACT WORK CAN BEGIN: Immediately Date: CONTRACT WORK MUST BE COMPLETED BY: Date 7/1/2007 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # PAINT LINE WORK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This is included in the bid This is not included in the bid Paint line work to be performed: (See attached map for locations) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TYPE OF LINE |  |  |  |  | WORK TO BE DONE |  |  | NOT APPLICABLE |  |  |  |  | PAINT COLOR |  |  |  |  |  |
| Private boundary |  |  |  |  |  |  |  |  |  |  |  |  | Blue |  |  |  | Other: |  |
| Sale boundary |  |  |  |  |  |  |  |  |  |  |  |  | Red |  |  |  | Other: |  |
| Sale cutting unit |  |  |  |  |  |  |  |  |  |  |  |  | Yellow |  |  |  | Other: |  |
| Stand type line |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Other: |  |
| Exclusions to mark and why: Steep hills will be excluded as indicated by the administering forester. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Standards for marking lines against private land: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| AREA CALCULATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This is included in the bid This is not included in the bid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UNIT |  | METHOD |  |  |  |  |  |  |  |  |  |  |  |  |  | STANDARD |  |  |
| Sale |  | GPS String Chain Other |  |  |  |  |  |  |  |  |  | map and acres |  |  |  | PDOP 6 |  |  |
| Payment Unit |  | GPS String Chain Other |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Stand |  | GPS String Chain Other |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Special Instructions: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TIMBER CRUISING SPECIFICATIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This is included in the bid This is not included in the bid Required Basal Area Factor: 10 20 Other: Cruise Line Directions: east/west |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRUISING UNITS |  |  |  |  |  |  | NUMBER OF PLOTS PER ACRE |  |  |  |  |  |  |  | SPACING (CHAINS) |  |  |  |
| stands 32,33,34 and 8 |  |  |  |  |  |  | 60 + one for every 4 acres |  |  |  |  |  |  |  | 5 X 5 |  |  |  |
|  |  |  |  |  |  |  | over 80 (approx 88 plots) |  |  |  |  |  |  |  | X |  |  |  |
| stand 7 and 37 |  |  |  |  |  |  | 32 plots |  |  |  |  |  |  |  | 3 X 3 |  |  |  |
| stand 40 |  |  |  |  |  |  | 30 plots |  |  |  |  |  |  |  | 3 X 3 |  |  |  |
| TOTAL NUMBER OF CRUISE POINTS |  |  |  |  |  |  | 150 plots |  |  |  |  |  |  |  |  |  |  |  |
| Cruise Special Instructions: Point sample cruise marked pulpwood trees only. Separate by mixed hardwood, aspen and basswood. Cruise residual basal area per plot. Marked sawlogs are to be tallied 100%. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Unless otherwise specified, the tally sheets used must be those supplied by the Forest, Mineral and Fire Management Unit. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TIMBER MARKING SPECIFICATIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This is included in the bid This is not included in the bid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Precise specifications to be delineated at pre-work meeting with DNR staff |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total Residual BA (Basal Area): 80 (Minimum 60 to Maximum 90) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TREATMENT |  |  |  |  |  |  |  |  |  |  | NUMBER |  |  |  |  |  |  | SIZE |
| Regeneration Gaps Per Acre |  |  |  |  |  |  |  |  |  |  | 0-3 |  |  |  |  |  |  | 30' crown to crown |
| Girdled Trees Per Acre |  |  |  |  |  |  |  |  |  |  | 1-3 |  |  |  |  |  |  | 8" dbh and up |
| PRODUCT |  |  |  | MARKING SYMBOL |  |  |  |  |  |  | MINIMUM DBH |  |  |  |  |  |  | TOP DIB |
| Sawlogs |  |  |  | X |  |  |  |  |  |  | 10" |  |  |  |  |  |  | 8" (9" DOB) |
| Sawbolts |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pulpwood |  |  |  | / |  |  |  |  |  |  | 4.6" |  |  |  |  |  |  | 4" |
| TYPE OF WORK |  |  |  | WORK TO BE DONE |  |  |  |  |  |  | NOT APPLICABLE |  |  |  |  |  |  | PAINT COLOR |
| Trees marked to leave |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Green Other |
| Trees marked to cut |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Orange Other |
| TALLY INTENSITIES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SPECIES / PRODUCT |  |  |  |  |  |  |  |  |  |  | RATIO |  |  |  |  |  |  |  |
| Sawlogs all species |  |  |  |  |  |  |  |  |  |  | 1 : 1 |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | 1 : |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | 1 : |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | 1 : |  |  |  |  |  |  |  |
| SPECIES TO FAVOR: Sugar Maple, Red Oak, Basswood |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SPECIES TO FAVOR: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SPECIAL MARKING INSTRUCTIONS: Contractor must paint dots on marked sawlog trees indicating the number of 8” sawlogs. Indicate trees to be girdled by two parallel lines at eye level completely around the tree. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Unless otherwise specified, the tally sheets used must be those provided by the Forest, Mineral and Fire Management Unit. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RESTRICTIONS: 1. Forest, Mineral and Fire Management Division must be able to identify who did what on the timber sale. 2. The contractor may not have more than two markers in one payment unit without prior approval. 3. Each marker should have an assigned area to mark (not strips). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PAINT: 1. Call Gaylord Field Office at (989) 732 - 3541, to make arrangements for paint. 2. Paint will be supplied from Gaylord Field Office. 3. Contractor’s work schedule must coincide with the DNR work schedule for paint supply. 4. The DNR will not be available on weekends or evenings. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DELIVERABLES: 1. A list of names that marked or cruised each payment unit. 2. A summary of paint used in quarts. 3. Tally summaries for each payment unit (one summary regardless of the number of markers) and all of the original tally sheets. 4. GPS files for any GPS work. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |