|  | Michigan Department of Natural Resources - Forest, Mineral and Fire Management Timber Sale Prescription |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GENERAL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date 4/11/2008 |  |  |  |  |  |  |  |  |  | Forest, Mineral and Fire Management Unit GRMUN (Indian River) |  |  |  |  |  |  |  |
| Timber Sale Number (if applicable) |  |  |  |  |  |  |  |  |  | Sale Name (or prescription name) Compartment 115 Contract |  |  |  |  |  |  |  |
| LOCAL CONTACT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name Shannon Harig |  |  |  |  |  |  |  |  |  | Telephone (231) 238 - 9313 ext. 4742 |  |  |  |  |  |  |  |
| Email Address Harigs@Michigan.gov |  |  |  |  |  |  |  |  |  | FAX (231) 238 - 6757 |  |  |  |  |  |  |  |
| Map of Project Area Attached |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LEGAL DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T36N R05W Section(s) 22,23 Description south 1/2 of both sections |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Year of Entry: 2008 Compartment(s):115 Stand Number(s): 12,13,15,38,41,46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| THIS TIMBER SALE CONTRACT IS BASED ON THE FOLLOWING ACREAGE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Estimated Acres: 175 Source: OI GPS Other Payment will be made on the basis of these estimated acres. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TREATMENT & OBJECTIVE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STAND # |  | COVER TYPE |  | ACRES |  | BA |  |  | TREATMENT |  |  |  | MANAGEMENT OBJECTIVE |  |  |  |  |
| 12 |  | M6 |  | 54 |  | 110 |  |  | Selection |  |  |  | Northern Hardwoods |  |  |  |  |
| 13 |  | A5 |  | 3 |  | 60 |  |  | Clearcut |  |  |  | Aspen |  |  |  |  |
| 15 |  | M6 |  | 1 |  | 110 |  |  | Selection |  |  |  | Northern Hardwoods |  |  |  |  |
| 38 |  | M9 |  | 58 |  | 120 |  |  | Selection |  |  |  | Northern Hardwoods |  |  |  |  |
| 41 |  | M6 |  | 56 |  | 110 |  |  | Selection |  |  |  | Northern Hardwoods |  |  |  |  |
| 46 |  | A5 |  | 3 |  | 60 |  |  | Clearcut |  |  |  | Aspen |  |  |  |  |
| PRESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Stands 12,15,38,41: - Reduce BA to 80 sq. ft. using single tree selection/crop tree release marking techniques. - Retain 2-3 12"+ DBH trees/acre of declining vigor(cull) to serve as future den trees. - When possible, release pockets of advanced hardwood regen by creating overhead canopy gaps. - Avoid marking under-represented species(cherry, elm, birch, etc.) to serve as retention. Stands 13,46: - Clearcut all species. Access West from Brutus 5 miles on Brutus Road. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DNR PREPARATION WORK TO BE DONE PRIOR TO CONTRACT WORK |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ESTIMATED DATE |  |  |
| All boundary and unit lines. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 05/19/2008 |  |  |
| CONTRACT WORK CAN BEGIN Immediately Date: 05/19/2008 CONTRACT WORK MUST BE COMPLETED BY Date 12/15/2008 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PAINT LINE WORK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This is included in the bid This is not included in the bid Paint line work to be performed: (See attached map for locations) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TYPE OF LINE |  |  |  |  | WORK TO BE DONE |  |  | NOT APPLICABLE |  |  |  | PAINT COLOR |  |  |  |  |  |
| Private boundary |  |  |  |  |  |  |  |  |  |  |  | Blue |  |  |  | Other: |  |
| Sale boundary |  |  |  |  |  |  |  |  |  |  |  | Red |  |  |  | Other: |  |
| Sale cutting unit |  |  |  |  |  |  |  |  |  |  |  | Yellow |  |  |  | Other: |  |
| Stand type line |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Other: |  |
| Exclusions to mark and why |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Standards for marking lines against private land |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| AREA CALCULATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This is included in the bid This is not included in the bid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UNIT |  |  | METHOD |  |  |  |  |  |  |  |  |  |  |  | STANDARD |  |  |
| Sale |  |  | GPS String Chain Other |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Payment Unit |  |  | GPS String Chain Other |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Stand |  |  | GPS String Chain Other |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Special Instructions: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TIMBER CRUISING SPECIFICATIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This is included in the bid This is not included in the bid Required Basal Area Factor: 10 20 Other: Cruise Line Directions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRUISING UNITS |  |  |  |  |  |  | NUMBER OF PLOTS PER ACRE |  |  |  |  |  |  | SPACING (CHAINS) |  |  |  |
| Stands 12,13,15,38,41,46 |  |  |  |  |  |  | 1 point/1 ac. |  |  |  |  |  |  | 3 X 3 |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |
| TOTAL NUMBER OF CRUISE POINTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cruise Special Instructions: Stands 12,15,38,41: Only pulpwood will be cruised, sawlogs are 100% tallied. White Ash, Basswood and aspen pulp must be tallied seperately. All other hardwood species can be combined as mixed hardwood pulp. Stands 13,46: Both sawlogs and pulp must be cruised in these stands. Sawlog species must be cruised seperately. Pulpwood must be cruised as described above. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Unless otherwise specified, the tally sheets used must be those supplied by the Forest, Mineral and Fire Management Unit. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| TIMBER MARKING SPECIFICATIONS |  |  |  |
| --- | --- | --- | --- |
| This is included in the bid This is not included in the bid |  |  |  |
| Precise specifications to be delineated at pre-work meeting with DNR staff |  |  |  |
| Total Merchantable Residual BA (Basal Area): 80 (Minimum 70 to Maximum 90) |  |  |  |
| TREATMENT |  | NUMBER | SIZE |
| Regeneration Gaps Per Acre |  | 1 per acre | 30' crown opening |
| Girdled Trees Per Acre |  | 1-2 | 12"+ |
| PRODUCT | MARKING SYMBOL | MINIMUM DBH | TOP DIB |
| Sawlogs | 2 orange slashes w/ dot | 12 inches | 10 inches |
| Sawbolts | N.A. | N.A. | N.A. |
| Pulpwood | orange slash | 4.6 inches | 4 inches |
| TYPE OF WORK | WORK TO BE DONE | NOT APPLICABLE | PAINT COLOR |
| Trees marked to leave |  |  | Green Other |
| Trees marked to cut |  |  | Orange Other |
| TALLY INTENSITIES |  |  |  |
| SPECIES / PRODUCT |  | RATIO |  |
| All sawlogs |  | 1 : 1 |  |
|  |  | 1 : |  |
|  |  | 1 : |  |
|  |  | 1 : |  |
| SPECIAL MARKING INSTRUCTIONS Individual stems within multiple stemmed clumps must either all be marked for removal or must all be left unmarked. Do not mark trees within 5' of the snowmobile trail. Do not mark Elm, Juneberry or Ironwood. Mark girdle trees with an orange "X" on both sides of tree. See administrator for marking instructions near stick nest. |  |  |  |
| Unless otherwise specified, the tally sheets used must be those provided by the Forest, Mineral and Fire Management Unit. |  |  |  |
| RESTRICTIONS - Forest, Mineral and Fire Management Division must to be able to identify who did what on the timber sale. - The contractor may not have more than two markers in one payment unit without prior approval. - Each marker should have an assigned area to mark (not strips). |  |  |  |
| PAINT - Call Shannon Harig at (231) 238 - 9313 ext. 4742, to make arrangements for paint. - Paint will be supplied from Indian River. - Contractor’s work schedule must coincide with the DNR work schedule for paint supply. - The DNR will not be available on weekends or evenings. |  |  |  |
| DELIVERABLES - A list of names that marked or cruised each payment unit. - A summary of paint used (in quarts). - Tally summaries for each payment unit (one summary, regardless of the number of markers) and all of the original tally sheets. - GPS files for any GPS work. |  |  |  |