|  | Michigan Department of Natural Resources - Forest, Mineral and Fire Management Timber Sale Prescription |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GENERAL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date 03/04/2008 |  |  |  |  |  |  |  |  |  | Forest, Mineral and Fire Management Unit Gladwin Management Unit |  |  |  |  |  |  |  |
| Timber Sale Number (if applicable) 73-012-08-01 |  |  |  |  |  |  |  |  |  | Sale Name (or prescription name) South Bourret Sale |  |  |  |  |  |  |  |
| LOCAL CONTACT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name Mike Hanley |  |  |  |  |  |  |  |  |  | Telephone (517) 243-2028 |  |  |  |  |  |  |  |
| Email Address hanleym@michigan.gov |  |  |  |  |  |  |  |  |  | FAX (989) 426-1454 |  |  |  |  |  |  |  |
| Map of Project Area Attached |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LEGAL DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T20N R2E Section(s) 35 & 36 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Year of Entry: 2008 Compartment(s):42 Stand Number(s): 41, 81 & 83 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| THIS TIMBER SALE CONTRACT IS BASED ON THE FOLLOWING ACREAGE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Estimated Acres: 131 Source: OI GPS Other Payment will be made on the basis of these estimated acres. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TREATMENT & OBJECTIVE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STAND # |  | COVER TYPE |  | ACRES |  | BA |  |  | TREATMENT |  |  |  | MANAGEMENT OBJECTIVE |  |  |  |  |
| 41 |  | E6 |  | 28 |  | 110 |  |  | Selection |  |  |  | Swamp Hardwoods |  |  |  |  |
| 81 |  | E6 |  | 62 |  | 80 |  |  | Selection |  |  |  | Swamp Hardwoods |  |  |  |  |
| 83 |  | E6 |  | 41 |  | 140 |  |  | Selection |  |  |  | Swamp Hardwoods |  |  |  |  |
| PRESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Stand 41: Aspen/Birch removal: Cruise but do not mark these species. Where needed, mark to cut Red Maple to facilitate logging of Aspen and Birch and to meet target residual of 70 BA/AC. - Stand 81: Aspen removal: Cruise but do not mark Aspen. Where needed mark to cut Red Maple to facilitate logging of Aspen and meet target residual of 80 BA/AC in low areas. - Stand 83: Aspen/Birch removal: Cruise but do not mark these species. Where needed mark to cut Red Maple to facilitate logging of Aspen and Birch and meet target residual of 90 BA/AC. Access: Via Lame Duck Truck Trail and various state forest two tracks. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DNR PREPARATION WORK TO BE DONE PRIOR TO CONTRACT WORK |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ESTIMATED DATE |  |  |
| None |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONTRACT WORK CAN BEGIN Immediately Date: CONTRACT WORK MUST BE COMPLETED BY Date 09/15/2008 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PAINT LINE WORK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This is included in the bid This is not included in the bid Paint line work to be performed: (See attached map for locations) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TYPE OF LINE |  |  |  |  | WORK TO BE DONE |  |  | NOT APPLICABLE |  |  |  | PAINT COLOR |  |  |  |  |  |
| Private boundary |  |  |  |  |  |  |  |  |  |  |  | Blue |  |  |  | Other: |  |
| Sale boundary |  |  |  |  |  |  |  |  |  |  |  | Red |  |  |  | Other: |  |
| Sale cutting unit |  |  |  |  |  |  |  |  |  |  |  | Yellow |  |  |  | Other: |  |
| Stand type line |  |  |  |  |  |  |  |  |  |  |  | Red |  |  |  | Other: |  |
| Exclusions to mark and why |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Standards for marking lines against private land N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| AREA CALCULATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This is included in the bid This is not included in the bid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UNIT |  |  | METHOD |  |  |  |  |  |  |  |  |  |  |  | STANDARD |  |  |
| Sale |  |  | GPS String Chain Other |  |  |  |  |  |  |  |  |  |  |  | PDOP < 7 |  |  |
| Payment Unit |  |  | GPS String Chain Other |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Stand |  |  | GPS String Chain Other |  |  |  |  |  |  |  |  |  |  |  | PDOP < 7 |  |  |
| Special Instructions: Contractor must GPS all stands and/or payment units seperately. Contractor must also provide a GPS file and hard copy of each stand printed at a scale of 1:15,840. See map for location and color of lines to be GPS'd. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TIMBER CRUISING SPECIFICATIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This is included in the bid This is not included in the bid Required Basal Area Factor: 10 20 Other: Cruise Line Directions North-South or East-West |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRUISING UNITS |  |  |  |  |  |  | NUMBER OF PLOTS PER ACRE |  |  |  |  |  |  | SPACING (CHAINS) |  |  |  |
| Stands 41, 81 & 83 |  |  |  |  |  |  | 1 |  |  |  |  |  |  | Evenly Spaced |  |  |  |
| TOTAL NUMBER OF CRUISE POINTS |  |  |  |  |  |  | 131 (approximately) |  |  |  |  |  |  |  |  |  |  |
| Cruise Special Instructions: - Total number of cruise points is estimate, based on estimated acres. Actual number should be based on 1 plot per acre, for acreage determined by GPS. - For stands 41, 81 and 83, ALL CRUISE POINTS MUST SHOW RESIDUAL BA. - Contractor MUST show plot locations for each stand on their GPS stand boundary map. Plot locations do NOT need to be GPS'd. - It is preferred that contractor use DNR tally sheets. Contractor may use their own tally sheets, with approval by Gladwin Forest Management Unit. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Unless otherwise specified, the tally sheets used must be those supplied by the Forest, Mineral and Fire Management Unit. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| TIMBER MARKING SPECIFICATIONS |  |  |  |
| --- | --- | --- | --- |
| This is included in the bid This is not included in the bid |  |  |  |
| Precise specifications to be delineated at pre-work meeting with DNR staff |  |  |  |
| Total Merchantable Residual BA (Basal Area): Varies by stand. See prescription section |  |  |  |
| TREATMENT |  | NUMBER | SIZE |
| Regeneration Gaps Per Acre |  |  |  |
| Girdled Trees Per Acre |  |  |  |
| PRODUCT | MARKING SYMBOL | MINIMUM DBH | TOP DIB |
| Sawlogs | Slash visible from all sides | 10” diameter class | 8” diameter class |
| Sawbolts |  |  |  |
| Pulpwood | Slash visible from all sides | 5” diameter class | 4” diameter class |
| TYPE OF WORK | WORK TO BE DONE | NOT APPLICABLE | PAINT COLOR |
| Trees marked to leave |  |  | Green Other |
| Trees marked to cut |  |  | Orange Other |
| TALLY INTENSITIES |  |  |  |
| SPECIES / PRODUCT |  | RATIO |  |
| N/A |  | 1 : |  |
|  |  | 1 : |  |
|  |  | 1 : |  |
|  |  | 1 : |  |
| SPECIAL MARKING INSTRUCTIONS For all stands Michigan DNR retention guidelines must be followed. Contractor will be required to mark and GPS a minimum of 3% and a maximum of 10% of stand area as retention. Retention may require marking leave trees with green paint, individually and/or in small groups. Implementation of the retention guidelines will be discussed on a stand by stand basis at the pre work meeting. |  |  |  |
| Unless otherwise specified, the tally sheets used must be those provided by the Forest, Mineral and Fire Management Unit. |  |  |  |
| RESTRICTIONS - Forest, Mineral and Fire Management Division must to be able to identify who did what on the timber sale. - The contractor may not have more than two markers in one payment unit without prior approval. - Each marker should have an assigned area to mark (not strips). |  |  |  |
| PAINT - Call Mike Hanley at (517) 243-2028, to make arrangements for paint. - Paint will be supplied from Gladwin field office. - Contractor’s work schedule must coincide with the DNR work schedule for paint supply. - The DNR will not be available on weekends or evenings. |  |  |  |
| DELIVERABLES - A list of names that marked or cruised each payment unit. - A summary of paint used (in quarts). - Tally summaries for each payment unit (one summary, regardless of the number of markers) and all of the original tally sheets. - GPS files for any GPS work. |  |  |  |