|  | Michigan Department of Natural Resources - Forest, Mineral and Fire Management Timber Sale Prescription |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GENERAL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date 8/21/08 |  |  |  |  |  |  |  |  |  | Forest, Mineral and Fire Management Unit Grayling |  |  |  |  |  |  |  |
| Timber Sale Number (if applicable) 728060901 |  |  |  |  |  |  |  |  |  | Sale Name (or prescription name) Compartment 206 mix |  |  |  |  |  |  |  |
| LOCAL CONTACT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name Lucas Merrick |  |  |  |  |  |  |  |  |  | Telephone (989) 348–6371 ext: 7446 |  |  |  |  |  |  |  |
| Email Address MerrickL@michgan.gov |  |  |  |  |  |  |  |  |  | FAX (989) 348-8825 |  |  |  |  |  |  |  |
| Map of Project Area Attached |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LEGAL DESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T27N R04w Section(s) 24 Description: SESE, N1/2SE, NE1/4, NW1/4-sec24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Compartment(s): 206 (YOE 2009) stands 3, 10, 13, 25, & 26 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| THIS TIMBER SALE CONTRACT IS BASED ON THE FOLLOWING ACREAGE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Estimated Acres: 197 Source: OI GPS Other Payment will be made on the basis of these estimated acres. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TREATMENT & OBJECTIVE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STAND # |  | COVER TYPE |  | ACRES |  | BA |  |  | TREATMENT |  |  |  | MANAGEMENT OBJECTIVE |  |  |  |  |
| 03 |  | R6 |  | 25 |  | 140 |  |  | Thinning |  |  |  | Red Pine |  |  |  |  |
| 10 |  | O9 |  | 63 |  | 80 |  |  | Seed Tree |  |  |  | Oak |  |  |  |  |
| 13 |  | A6 |  | 39 |  | 70 |  |  | Final Harvest |  |  |  | Aspen |  |  |  |  |
| 25 |  | O6 |  | 57 |  | 120 |  |  | Final Harvest |  |  |  | Oak |  |  |  |  |
| 26 |  | R6 |  | 13 |  | 130 |  |  | Thinning |  |  |  | Red Pine |  |  |  |  |
| PRESCRIPTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Stand 03-Thinning-Mark to cut to a residual of 80 BA - Stand 10-Seed Tree Mark-to-Leave to a residual of 20 BA with retention - Stand 13-Final harvest with retention - Stand 25-Final harvest with retention - Stand 26- Thinning-Mark to cut to a residual of 70 BA Access: From the entrance of the Grayling High School go north 1.5 miles on Old U.S. 27, then turn right follow two-track to the snowmobile trail. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DNR PREPARATION WORK TO BE DONE PRIOR TO CONTRACT WORK |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ESTIMATED DATE |  |  |
| CONTRACT WORK CAN BEGIN Immediately Date: 10/1/2008 CONTRACT WORK MUST BE COMPLETED BY July 31, 2009 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PAINT LINE WORK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This is included in the bid This is not included in the bid Paint line work to be performed: (See attached map for locations) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TYPE OF LINE |  |  |  |  | WORK TO BE DONE |  |  | NOT APPLICABLE |  |  |  | PAINT COLOR |  |  |  |  |  |
| Private boundary |  |  |  |  |  |  |  |  |  |  |  | Blue |  |  |  | Other: |  |
| Sale boundary |  |  |  |  |  |  |  |  |  |  |  | Red |  |  |  | Other: |  |
| Sale cutting unit |  |  |  |  |  |  |  |  |  |  |  | Yellow |  |  |  | Other: |  |
| Retention islands & leave clumps |  |  |  |  |  |  |  |  |  |  |  | Green |  |  |  | Other: |  |
| Exclusions to mark and why: For retention: In stand 10 leave a total of 5% of the stand in 0.25 acre islands scattered throughout the stand. In addition mark all red pine trees larger than 20 inches at DBH with a single green paint line that encircles the trees. In stand 13, leave a total of 3% of the stand acres in scattered islands ranging in size from 0.25 - 0.5 acre along the snowmobile trail. In stand 25 leave a total of 8 % of the stand in 0.25 – 0.5 acre islands scattered throughout the stand avoid leaving any aspen in the retention islands, in addition leave one HEALTHY oak every two acres with a single green paint line that encircles the trees. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Standards for marking lines against private land N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| AREA CALCULATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This is included in the bid This is not included in the bid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UNIT |  |  | METHOD |  |  |  |  |  |  |  |  |  |  |  | STANDARD |  |  |
| Sale |  |  | GPS String Chain Other |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Payment Unit |  |  | GPS String Chain Other |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Stand |  |  | GPS String Chain Other |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Retention Island |  |  | GPS String Chain Other |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Special Instructions: GPS files must provide acreage for the total sale area and for each stand, payment unit and retention island. Unit lines must be recorded as line features. Retention islands must be recorded as area features. See APPENDIX A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TIMBER CRUISING SPECIFICATIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This is included in the bid This is not included in the bid Required Basal Area Factor: 10 20 Other: Cruise Line Directions East/West |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CRUISING UNITS |  |  |  |  |  |  | NUMBER OF PLOTS PER ACRE |  |  |  |  |  |  | SPACING (CHAINS) |  |  |  |
| 1 (stand 03) |  |  |  |  |  |  | 1 |  |  |  |  |  |  | 3.3 X 3 |  |  |  |
| 2 & 3 (stand 10) |  |  |  |  |  |  | 0.8 |  |  |  |  |  |  | 3.3 X 4 |  |  |  |
| 4 (stand 13) |  |  |  |  |  |  | 1 |  |  |  |  |  |  | 3.3 X 3 |  |  |  |
| 5 & 6 (stands 25) |  |  |  |  |  |  | 0.9 |  |  |  |  |  |  | 3.3 X 3.5 |  |  |  |
| 7 (stand 26) |  |  |  |  |  |  | 1 |  |  |  |  |  |  | 3.3 X 3 |  |  |  |
| TOTAL NUMBER OF CRUISE POINTS |  |  |  |  |  |  | 117 |  |  |  |  |  |  |  |  |  |  |
| Cruise Special Instructions: Cruise all species separately. Separate cruise data by stand. See APPENDIX 8 (will be provided upon request) and APPENDIX B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Unless otherwise specified, the tally sheets used must be those supplied by the Forest, Mineral and Fire Management Unit. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| TIMBER MARKING SPECIFICATIONS |  |  |  |
| --- | --- | --- | --- |
| This is included in the bid This is not included in the bid |  |  |  |
| Precise specifications to be delineated at pre-work meeting with DNR staff |  |  |  |
| Total Merchantable Residual BA (Basal Area): see below (Minimum see below to Maximum see below) |  |  |  |
| TREATMENT |  | NUMBER | SIZE |
| Regeneration Gaps Per Acre |  |  |  |
| Girdled Trees Per Acre |  |  |  |
| PRODUCT | MARKING SYMBOL | MINIMUM DBH | TOP DIB |
| Sawlogs |  |  |  |
| Sawbolts |  |  |  |
| Pulpwood |  |  |  |
| TYPE OF WORK | WORK TO BE DONE | NOT APPLICABLE | PAINT COLOR |
| Trees marked to leave |  |  | Green Other |
| Trees marked to cut |  |  | Orange Other |
| TALLY INTENSITIES |  |  |  |
| SPECIES / PRODUCT |  | RATIO |  |
|  |  | 1 : |  |
| SPECIAL MARKING INSTRUCTIONS: In stand 3, mark-to-cut with orange paint to a residual BA of 80 (min 70 to max 90). Removing the majority of jack pine, but leaving some for retention. Also marking the suppressed red pine around the oak trees. In stand 10, mark-to-leave with green paint to a residual BA of 20 (min 10 to max 30), concentrating on cutting the smaller diameter oak, and marking all red pine trees larger than 20 inches at DBH with a single green paint line that encircles the tree to leave. In stand 26, mark-to-cut with orange paint to a residual BA of 70 (min 60 to max 80), taking out some of the economically mature pine, but marking for future poles. |  |  |  |
| Unless otherwise specified, the tally sheets used must be those provided by the Forest, Mineral and Fire Management Unit. |  |  |  |
| RESTRICTIONS - Forest, Mineral and Fire Management Division must to be able to identify who did what on the timber sale. - The contractor may not have more than two markers in one payment unit without prior approval. - Each marker should have an assigned area to mark (not strips). |  |  |  |
| PAINT - Call Lucas Merrick at (989) 348 - 6371 ext: 7446, to make arrangements for paint. - Paint will be supplied from the Grayling Field Office. - Contractor’s work schedule must coincide with the DNR work schedule for paint supply. - The DNR will not be available on weekends or evenings. |  |  |  |
| DELIVERABLES - A list of names that marked or cruised each payment unit. - A summary of paint used (in quarts). - Tally summaries for each payment unit (one summary, regardless of the number of markers) and all of the original tally sheets. - GPS files for any GPS work. - Cruise data entered into POSALE and exported onto a disk. Use the sale name and number from page 1. |  |  |  |